Quotation details

Overall Cost

Item Amount

Construction Cost INR 76,76,800/-

Architecture, Other Design & Operation Cost INR 4,22,224/- (details below)

Final Cost INR 80,99,024/-

Construction Details

Plot Area = 3200.00

Duplex -Ground Floor => 400.00 sq.ft (1 units of carParking) + 1000.00 sq.ft (1 units of

1bhk)

Duplex -1st Floor => 1200.00 sq.ft (3 units of room) + 300.00 sq.ft (1 units of hall)

Headroom Area = 100.00

Package Built Up Area = 2600.00 sq.ft

Total Built Up Area = 3000.00 sq.ft

As there is a duplex in the plan, Changes in the floor built up area will change the

project cost.

<b>Additions to Package:</b>

240 rft added for Compound Wall - 5 ft. height with 4 inches Solid Concrete Blocks with

SSM base of 1.5 ft. width x 2 ft. height and neccessary foundation up to 2', which include

excavation, PCC, SSM, masonary, plastering, painting, backfilling etc.. complete on top of

package

100 sqft added for MS Main Gate - 1.5kg/sft' (Hinged)-5' height with Enamel Paint-

Material & Labour on top of package

12600 sqft added for Additional foundation depth for every 1 ft. depth beyond the

specification of the package depth - Only for builtup area-Excl. Steel Cost-Material &

Labour-Incl. Backfilling on top of package

* Note: Kindly check the Open Area in the quote as the total Open Area is greater than

20% and an increase/decrease in the areas will affect the price.

Architecture, Other Design & Operation Cost

Item Amount

Architectural Design Fee (2.5% of Construction

Cost)

INR 1,91,920/-

Includes:

Onboarding, Digital Survey, Soil Testing, Site

Inspection and Feasibility, 2D Floor Plan (Iterations)

Other Design & Pre-Construction Fee (3% of

Construction Cost)

INR 2,30,304/-

Includes:

Structural Drawings, Framing Layout,, Doors &

Window Schedule, 3D Elevation Drawings,

Contractor Allocation, Quotation Preparation,

Contracting Charges, Bar Bending Schedule

Total Architecture, Other Design & Operation Cost INR 4,22,224/-

Total Construction Cost Breakup

Item Rate Quantity Cost(INR)

Ground Floor + 1st Floor Specification - Premium Package 2350.0 per

sqft

2600.0

sqft

61,10,000

Ground Floor 1 units of carParking 1527.5 per

sqft

400 6,11,000

Compound Wall - 5 ft. height with 4 inches Solid Concrete

Blocks with SSM base of 1.5 ft. width x 2 ft. height and

neccessary foundation up to 2', which include excavation,

PCC, SSM, masonary, plastering, painting, backfilling etc..

complete (Refer Annexure A1)

1695 per rft 240 4,06,800

MS Main Gate - 1.5kg/sft' (Hinged)-5' height with Enamel

Paint-Material & Labour (Refer Annexure A2)

450 per sqft 100 45,000

Additional foundation depth for every 1 ft. depth beyond

the specification of the package depth - Only for builtup

area-Excl. Steel Cost-Material & Labour-Incl. Backfilling

(Refer Annexure A3)

40 per sqft 12600 5,04,000

Total Construction Cost 76,76,800/-

Ground Floor + 1st Floor Specification

Package Name Premium Package for Turnkey House Construction in Bangalore

Package Rate INR 2350.0 per sqft

Package Built-up Area 2600.0 sqft

Designs & Drawings

- Architectural Layout | 2D

- Basic Elevation

- Structural Design

- 3D Elevation

- All the designs will be provided in non editable PDF format

- Only External 3D views as visible from the adjacent roads will be provided

- Electrical Drawings

- Plumbing Drawings

Structure

- - RCC Framed Structure in M20 / M25 grade Concrete in 20mm & 40mm Aggregates or as

advised in the RCC Design Mix as per the Structural designer. - Height of each floor shall be 10'0"

from Finished Floor level to Finished Floor level, i.e.measured from top of the floor finish in one

floor to the top of the floor finish in the next floor above that. Internal clear heights will be less

by the thickness of the slab and the thickness of the finished floor.

- RCC Design Mix - ACC or Ultratech M20 / M25 or As per the structural designer

recommendation

- Aggregates - 20mm & 40mm

- Cement - ACC or Ultratech or Ramco Supercrete of 43 or 53 grade of prices at Rs. 400/- per

Bag

- Steel - Jindal Panther or Vizag or Indus of price at Rs. 73,000/- per MT

- Steel in the package is assumed at 2.9kg per Sqft (Any Steel Quantity over and above this will

be chargeable as per actuals based on the recommendations provided by the Structural

Designer, and labour charges shall be applicable on such costs.

- Height of each floor shall be 10'0" from Finished Floor level to Finished Floor level

- Blockwork Masonry in Standard Solid Concrete Blocks with Plaster as per the design 4" for

internal Partitions and 6" for External Walls . 6 inch for Exterior Walls is assumed at 40 (+/-5 )

per Block & 4 inch for Interior Walls is assumed at 30 (+/-5 ) per Block

- 	 The Foundation has been assumed as Isolated footings in regular Soil of Bearing Capacity

180kN/m^2 and above and upto a depth of 5 feet. For soil conditions requiring specialised

foundation design or/and additonal depth of excavation over and above 5 feet, additional

charges will be applicable. For every 1feet of additional excavation, INR 35/sqft will be charged

for entire site area.

- In case the Site level is more than 6" below the Road level, additional filling charges will be

applicable at 48,000 for 1' for a 30'X40' plot size and prorated thereafter

- The Plinth level will be Provided at a height of 1 feet and 6 inches above the existing Ground

Level. PCC of 100mm shall be Provided at the Plinth Level below the Ground floor Flooring. Any

increase in height of the Plinth thereafter shall be chargeable at 60 Rs per sqft for 6" increase

and size stone masonry at 160/cu.ft. will be charged for raising plinth height by 1ft and prorated

thereafter. SSM below the plinth beam is chargeable additional at 160 per CFT

- Chicken mesh shall be used near beam, column and masonry wall junction and at the Chasing

for Electrical Conduits wherever the chasing is more than 2” in depth and width

- Weather proof course shall be provided at terrace of Dr. Fixit brand or equivalent.

- Lintels shall be provided only over openings with bearing of 6" into the edge of the openings.

For providing concrete band all around, additional Charges shall be applicable at INR200 per

running feet.

- Curing compound of Sika Antisol/ equivalent brand shall be used in the external plastering.

- M-Sand shall be used for Masonry and P-Sand shall be used for Plastering

- Column size being considered is (6" X 1'6")- Column Upgradation from 6" Column to 8" Column

shall be charged additional at 30 Rs per Sqft and Upgradation to 9" Column shall be charged

additional at 35 Rs per Sqft On Total Built-up Area -Excl. Steel Cost

Kitchen

- For every one dwelling unit one kitchen of size upto 80 sft is included. Additional dwelling units

and larger kitchens shall be charged at a Pro-rata basis

- Ceramic wall tiles will be provided upto 2'-0" above the Kitchen Counter top of - upto 80 per

Sft

- Kitchen counter top of width, 2'-0" in 18mm Granite of - upto 200 per Sft is included. Double

layer nosing at the edge in the front has been included along with polishing

- Single bowl Stainless Steel Kitchen Sink of - upto 8000 shall be provided. Kitchen faucet of

upto 3500 along with accessories of Parryware/Hindware/Jaquar or equivalent make shall be

provided

- Provision shall be made for One inlet near Kitchen Sink for the Water Purifier or as per Design

- A Bib Cock of Jaquar or equivalent make of INR 800 shall be provided in the Utility Area -

Provision of Exhaust Fan is included. The Exhaust fan fixture is not included.

- Ceramic Wall Tiles ( 2 feet above kitchen slab ) - Upto 80 per Sqft

- Main Sink Faucet - Upto 3500

- Any Other Faucet & Accessories - Jaquar/Parryware/Hindware

Bathroom

- One bathroom is included per 500 sft of package built up area. Additional bathrooms will be

charged on a Pro-rata basis

- Bathroom Ceramic wall tiles will be provided upto 7'-0" above the Finished Floor Level of -

upto 80 per Sft

- Bathroom Flooring - Anti-skid Tiles of value upto 80/- per sqft

- - Gratings shall be sealed with cementitious grout of Roff/ equivalent brands

- Two nos. of 6 inches rainwater discharge pipes shall be provided per 1000 sqft of plot area.

Additional Rain Water Pipes will be charged on a Pro-rata basis

- Sewer line connection to the main drain line is included in the package up to 10 feet from the

building. Any road cutting permissions or charges are not included.

- Sanitary ware & CP Fittings - Upto 70000 per 1000 sqft of construction of Jaquar or

equivalent makeOne number floor mount EWC, Health Faucet, Wash Basin, Basin Mixer,

Overhead Showerwith 2 in 1 Shower wall Mixer of Jaquar/Hindware/Parryware make is included.

Additional Sanitary Ware and CP Fittings will be charged on a Pro-Rata Basis.

- Sanitary ware and CP fittings of - upto 70,000 are included per 1000 sft of Package Built Up

area. Additional Sanitary Ware and CP Fittings will be charged on a Pro-Rata Basis. One number

EWC, Health Faucet, Wash Basin, Basin Mixer, Overhead Shower with 2 in 1 Shower wall Mixer is

included.

- Provision of Exhaust Fan is included. Exhaust fan fixture is not included.

- Mirror, Soap Dish, Towel Rail - Worth 7000 till 1000 ft of construction

- Pipe networking of Solar water heater provision shall be provided

Doors & Windows

- The total area of opening for windows and ventilators including frames is taken as 10% of the

package built up area. Windows and Ventilators for any area over and above this shall be

chargeable.

- Windows - UPVC Windows with glass and mesh shutters 3 track (3 track with 1 mesh) of NCL

Veka brand or Prominance or V-Tech or Greentech worth 700/- per sft

- Window Grills of upto 195 per sft shall be provided of 10mm thick MS rods with maximum

spacing 125mm between rods in either direction finished with enamel paint

- - The timber used for doors/door frames shall be well seasoned and free from knots, warps,

sap, cracks, and other defects. All joints should be strong, accurately fitted, and glued and shall

be provided with two coats of paint

- Chajjas shall be provided for all Windows with 1 feet outward projection and 6 inch bearing on

each side of the window

- Main Door - Teak Door with teak frame of 5inch by 3.5 inch, worth 40000 including fixtures

- 1 Main door shall be provided per dwelling unit. The main door shall be 32mm thick of Size

3'6"X7' of - upto 40,000 including frame in African Teak or equivalent. The frame thickness will

be 3"X5". The Main door price includes 2 numbers brass tower bolts, 3 brass hinges, Lock of upto

2000 of Europa/Godrej make and 1 Wall mounted Magnetic Door Stopper

- Internal Doors shall be 32mm thick Membrane/Flush doors of Size 3'0"X7' of - upto 13,000

including frame in Sal Wood. The frame thickness will be 2.5"X4". The price includes 2 numbers

MS/Aluminium tower bolts, 3 MS hinges and 1 Wall mounted Magnetic Door Stopper

- Bathroom Doors shall be 32mm thick Waterproof Flush doors / WPC doors of Size 2'6"X7' of -

upto 13000 including frame in Sal wood. The frame thickness will be 2.5"X4". The price includes

2 numbers MS/Aluminium tower bolts, 3 MS hinges and 1 Wall mounted Magnetic Door Stopper

- Maximum 1 Bedroom door and 1 Bathroom door shall be provided for 500 sft of package built

up area. Additional doors shall be provided only in the incremental step of 500 sft of package

built up area. No intermediate doors are included.

- Puja Room Door - Burma Teak along with teak frame of 5inch by 2.5 inch, worth 28000

including fixtures for every 2000 sft of package built up area

Painting

- The exteriors will be painted with one coat Asian Primer and two coats of Apex Exterior

Emulsion paint or equivalent

- The interior walls and Ceiling shall be painted with 2 coats JK Putty, 1 Coat of Primer and 2

coats - Apcolite Premium Emulsion or equivalent. The choice of colours is restricted to upto 2 in

a room and upto 6 overall. Any additional colours will be chargeable

- All MS Steel grills and railings shall be painted with an anti corrosion coat with a final coat of

Enamel Paint.

- Interior Painting - JK Putty + Apcolite Premium Emulsion or equivalent

- Exterior Painting - Asian Primer + Apex Exterior Emulsion Paint or equivalent

Flooring

- Living and Dining room flooring shall be in 2'X2' Vitrified Tiles or Granite of - upto 140 per sft

- Other Rooms and Kitchen flooring shall be in 2'X2' Vitrified Tiles or Granite of - upto 120 per

sft

- Staircase flooring shall be in Sadderahalli Granite of - upto 110 per sft

- Parking area flooring shall be in Anti-skid tiles of - upto 70 per sft

- Balconies and open areas shall be in Anti-skid tiles of - upto 80 per sft. Terrace tiles shall be

additionally charged

- Skirting shall be of matching Vitrified Tiles or Granite not less than 100mm high

- Wall dado will be in 18"X12" ceramic tiles of - upto 80 per sft

- The following Quality standards and inclusions/exclusions shall be made note of and must be

taken care of at the time of execution

- Wall tiles shall be fixed with tile adhesive. Joints shall be cleaned thoroughly and grouted with

site made grout cement grout of Roff/ Weber/ equivalent brands

- Ceramic tiles for walls shall be free from cracks, spots, grazes, chipped edges and corners.

Variation in size shall be limited to +/- 1.5 mm. Thickness shall be as specified, but in no case

shall they be less than 6 mm

- Flooring/dado/skirting must be laid level/to plumb

- Grouting for tiles in the Balconies, terrace (if opted) shall be done with cementitious grout of

Roff/ equivalent brands

- Plinth protection upto two feet wide is included in the package

- Granite if used shall be hard, sound, free from cracks, cavities, holes, patches of injurious veins,

weathered portions, flaws, etc. Granite used should have a minimum thickness of 19 mm

- Labour charges for cutting, laying, polishing, nosing, chamfering of granite shall be extra

- Labour charges for laying of tiles shall be extra if tile of size greater than 2ft by 4ft is used.

- Additional charges shall be applicable for laying Indian marble of upto 70 per sft

- Additional charges for laying Granite shall be 18 per sq.ft. Additional charges of 30 per

running ft for buffing or chamfering/nosing if applicable

- Additional charges shall be applicable for laying Italian marble of upto 70 per sft. Over and

above the laying charges, an additional chemical treatment charge of 30 per sft shall be

applicable

Electrical

- All wiring shall be done with fire proof wires of Finolex or Anchor or Havells.

- Switches & Sockets - Legrand Mylinc or Havells Coral or Roma

- The electrical points shall be provided as per the following distribution

- Living/Dining/Family Room shall be provided with 2 numbers fan Point, 4 numbers Light Point,

4 numbers 5Amp Switch and Sockets and 1 number TV Cable Point

- Other Rooms shall be provided with 3 numbers of light points, 1 number of fan point, 4

numbers of 5Amp switch and socket in the TV/Study or dresser area as the case may be as per

design and 1 number 16Ampswitch and socket for air conditioning

- Kitchen shall be provided with 2 numbers of light points, 5 numbers of 5/15Amp (as the case

may be based on Appliance Specifications) switch and socket one each for refrigerator,

mixer/grinder, oven, water purifier and one above the Kitchen platform

- 1 number calling bell shall be provided. More call bell points per dwelling Unit shall be charged

additionally

- Bathroom shall be provided with 2 numbers of light point, 1 number of geyser point, 1 number

of 5Amp Switch and Socket near the wash basin

- 1 number light point each shall be provided with a two way switch connection for each room,

living room and staircase

- Corridor/passage/portico/parking/utility/terrace/external stairs/balcony/space around

house/any other built up area shall be provided with 5 numbers light points and 5 numbers

5Amp switches and sockets at suitable locations per 1000 sqft of package built up area

- All wiring shall be done with fire proof wires of Finolex or Anchor or Havells.

- Provision for 1 nos. of UPS Wiring shall be made for every 2,000 sft of package built up area.

- One MCB board of Indoasian make shall be provided

- One earthing provision shall be provided for one dwelling unit per 1000 sft of package built up

area

- Main panel board shall be in the Owner's scope including road cutting charges and main-line

from pole to panel board

Miscellaneous

- A Sintex Double layered overhead tank of - 2000L shall be provided. Any Additional capacity

shall be chargeable at INR 9 per L. Platform for the OHT shall be charged additional based on the

design and specifications

- A Double layered overhead tank of - 2000 Ltrs. shall be provided. An Additional capacity shall

be chargeable at INR 10 per Ltrs. Platform for the OHT shall be charged additional based on the

design and specifications

- Underground Sump - 7000 litres

- An Underground sump of - 7000 Ltrs. shall be provided with Concrete block masonry. PCC

bed concrete of minimum 125mm thickness shall be provided at the base. The inside of the wall

shall be provided with chicken mesh plastering. MS cover plate of dimension 600 mm x 600

mm shall be provided. Water Proofing chemical be applied to mortar for plastering. Additional

capacity of Underground Sump tank shall be charged at 9 per Ltrs. RCC Underground sump

shall be charged at 19 per Ltrs. over and above the base cost

- Staircase Railing - SS (Stainless) Railing of SS 304 grade profiles

- One simple pattern Staircase Railing only on one side of the staircase be in SS of 304 grade -

upto 1800 per rft for material, painting, welding, fixing, and finishing. For more Staircases or for

railing on the other side of staircase, additional charges shall apply.

- Compound Wall shall be charged additional. If required additional, the compound wall shall be

in solid cement block masonry with 5 feet height from the ground level and 115 mm in thickness.

Buttress/columns shall be provided for every 10 feet length of the compound. Expansion joints

may be provided wherever necessary. The compound wall shall be constructed on size stone

masonry foundation of minimum thickness 450mm and minimum depth 2 feet. The foundation

shall be laid on compacted hard soil. Bed concrete of minimum thickness 80mm shall be

provided below the foundation. Compound wall shall be plastered and painted as per the

plastering and painting specifications

- Parapet wall shall be in Blockwork and shall be provided to a height of 3 feet and 150mm

thickness. Buttresses shall be provided at every 10 feet of length

- Balcony and Utility railings along with the Terrace parapet railings if opted for shall be charged

additional at - upto 300 per sft for MS railings with material, painting, welding, fixing, and

finishing. More complex patterns shall be chargeable as per the design

- A Septic tank in blockwork, if applicable shall be charged at 13/Ltrs. For RCC septic tank

additional charges 23/Ltrs. shall be applicable

- Elevation of the building on all sides shall include basic plastering and external painting with no

cladding/ grooves/ cuts/ projections or any other aesthetic elements like cladding, pergola, etc

- Antitermite treatment is additional and may be provided at an extra charge of - 16/sqft

- Rain water harvesting (7ft depth, rings installation, course and fine aggregate bed) shall be

provided as per BBMP norms at additional - INR 30000

- External Drain covers in Natural stone of width 100mm of upto - 300 per sft shall be provided

at an additional charge

- Lofts inside rooms and kitchen are not part of standard package, it is provided at 350/sqft

- Puja Room Door - Burma Teak along with teak frame of 5inch by 2.5 inch, worth 28000

including fixtures for every 2000 sft of package built up area

Customisation Costs

- SSM below plinth is not part of package

- External Drain cover (100mm) cost 300 per sq.ft using Natural Stone

- Italian Marble chemical treatment cost at 30 per sq.ft (Excluding laying cost)

- Additional capacity for RCC underground sump at 18/Ltrs.

- Additional capacity for overhead tank at 9 per Ltrs.

- Additional capacity for blockwork underground sump is 9/Ltrs.

- Additional cost of laying Indian marble laying is 70 per sq.ft extra

- Blockwork septic tank cost is Rs 13/L and concrete septic tank cost is Rs 23/L

Terms and conditions

- All construction work comes with a one-year warranty limited to 1% of the Contract value. The

date of start of warranty is the date of project completion including snags as entered in the

'Hand Over Letter'

- Any work additional to the scope mentioned above shall be chargeable. Wherever such

additional work is not described in this contract shall be quoted by the Contractor in writing

and accepted by the Owner in writing. In case there's a difference in opinion of the correctness

of the quote provided by the contractor, the Consultant's decision shall be final

- All rates are inclusive of GST

- Wherever the word Equivalent has been used it means approximately the same price

- All power and water for construction to be provided by the client at free of cost

Reduced items from opted package

Site Conditions Impacting Cost

* The quote is prepared based on standard criteria mentioned. Any Observed changes beyond the standard

details, in the site details mentioned will be charged as per requirement.

S.No. Check Points UOM Standard

1 Offsets provided by left,right and back neighbours? Ft >3 Ft

2 Electric Pole/Transformer location near to site? Yes/No No

3 Existing infrastructure on site which needs to be

demolished?

Yes/No No

4 Atleast 64 sqft area available for labour shed? Yes/No Yes

5 Is labour allowed to stay at site? Yes/No Yes

6 10% of site area for storage of materials? Yes/No Yes

7 If Yes storage area is available, is that available near the

site?

Yes/No Yes

8 What is the road width in front of the plot (excluding

footpath)?

Ft >20ft

9 Road traffic conditions at the plot? High/Medium/

Low

Low

10 Difference in level of the site with respect to the centre

of the road?

Ft 0 Ft

11 Is there any slope in the plot? Yes/No No

12 Is there any rocky terrain present on site? Yes/No No

13 Is there any rocky terrain present on site, shared with

neighbouring building?

Yes/No No

14 Water table high Yes/No No

15 Is Site contour provided Yes/No Yes

16 Drainage Line marking and information provided Yes/No Yes

Points to Note :

1. This is a Specification and not a Contract. This will be converted into a contract once the customer

starts off the project with company.

2. The quotation is marked as final (will not change) only when a minimum 5% of the total project value is

received as a booking amount. Post this, it is valid for the next 30 days.

3. The quotation value will change if the customer adds/removes/updates any specification.

4. At Company we have zero tolerance for delays - In the contract there are strict compensation clauses

for any delay in a project by the professional (contractor), where the customer is compensated.

Compensation calculation table is provided for your reference as part of this quotation.

5. Quotation is prepared based on optimal structural designs based on all relevant IS codes for

construction. Customised designs may levy an additional fee.

6. The total compensation for delay shall be a lower value between - the last milestone value or 2.55% of

total project value.

7. The following activities are taken up by our Construction Professionals post the payment of the booking

amount: Site Reconnaissance, Soil Testing, Structural Drawings, Approval of Structural Drawings, Contract.

Drafting and Site Meeting. It takes about 64 days to complete all these activities and work on the site can

commence only post completion of all these activities.

8. The construction will be carried out as per the specifications, dimensions and quantities as mentioned

in this quotation. Differential charges will be applicable for any change in specifications, dimensions or

quantities as per the actual calculations and rates mutually agreed between and with the written consent

of Customer and Contractor.

9. All such changes shall be routed through Company and the value of this Quotation for all purposes

would be considered inclusive of the differential charges for these changes. All terms and conditions

defined in the vendor onboarding agreement between the Contractor and Company will apply to the

revised value of the quotation.

10. All payments are required to be transferred to the company's dedicated escrow account number

allocated to the owner. No cash payments will be accepted. Any cash payments made by the owner to

anyone is the owner's responsibility and the company will not be held responsible for any losses thereof.

11. Price Guard: Your quoted price is secured for 2 months from the date of this quotation. After this

quotation has expired, a new quotation will be provided based on prevailing market rates.

12. Refunds shall be governed by a slab-wise structure based on the Project Value (NDV). The service fee

to be retained by us  shall be in accordance with the schedule provided below:

Service Project Value < 1Cr Project Value > 1Cr

Architectural Design Stage

Onboarding 10,000 10,000

Soil Test 20,000 20,000

Digital Survey 10,000 10,000

Floor Plan (iterations) 2.5% (after deducting the

above fixed charges)

2.0% (after deducting the

above fixed charges)

Stage Total 2.5% 2%

Other Design & Pre-Construction Stage

Structural Drawings 1% 0.75%

Framing Layout, Doors and Window

Schedule

0.5% 0.5%

Bar Bending Schedule 0.25% 0.25%

Final Quotation Preparation 0.25% 0.25%

Contractor Allocation & Contract Signing 0.25% 0.25%

3D Elevation Drawings 0.75% 0.75%

Stage Total 3.0% 2.5%

Total 5.50% 4.50%

13. Temporary electrical and water connections are not included in the contractor's scope of work.

Annexure

A1. Compound Wall - 5 ft. height with 4 inches Solid Concrete Blocks with SSM base of 1.5 ft. width x 2 ft.

height and neccessary foundation up to 2', which include excavation, PCC, SSM, masonary, plastering,

painting, backfilling etc.. complete

- Compound Wall - 5 ft. height with 4 inches Solid Concrete Blocks with SSM base of 1.5 ft. width x 2 ft.

height and neccessary foundation up to 2', which include excavation, PCC, SSM, masonary, plastering,

painting, backfilling etc.. complete

A2. MS Main Gate - 1.5kg/sft' (Hinged)-5' height with Enamel Paint-Material & Labour

- MS Main Gate - 1.5kg/sft' (Hinged)-5' height with Enamel Paint-Material & Labour

A3. Additional foundation depth for every 1 ft. depth beyond the specification of the package depth -

Only for builtup area-Excl. Steel Cost-Material & Labour-Incl. Backfilling

- Additional foundation depth for every 1 ft. depth beyond the specification of the package depth -

Only for builtup area-Excl. Steel Cost-Material & Labour